https://www.youtube.com/watch?v=dj-i8vMkm8w

- Sprocket +8 teeths for more torque lower speed

- Smaller carb from TT600 1st gen 1983 ? More torque less idle speed

- change air intake for underseat to help engine cooling

- (maybe new cylinder head with fins)

- piston clearance

- larger pivoting foot pegs

- larger thicker rims (alloy breaks often)

- wiring loom XT600K post 90-94 no relay and same light same stator

- 2nd CDI

- do not electricity to start

- 2nd wiring loom to start with a stripe to recognize

- 2 regulators on aluminium plate

- 2 ignition coil

- bearing for electric starter ‘91 3AJ model

- SAX 400E starter new

- Gearbox strongest from TTR rubber on rear wheel

- no relay side stand or clutch

- Barkbusters clutch protection handlebar

- no fuel pomp so last 2L are under carburator, no fuel gauge

- Nissin double pistons brakes from XT

- first copy of yamaha piston

- lower the compression high around 8:5 or 9:1 not more

- piston intake burnt because intake rubber is gone too much air then overheating

- check rubbers intake 69€

https://www.youtube.com/watch?v=_-ZOK0Gce3w

- Metal bearing for rear shock made custom

- soft rubber to keep rear shock clean x2 (axle + cover)

https://www.youtube.com/watch?v=DYY6BvnAKwc

- plastic cover metal frame protection for side panels

- heat protection tape mod

- TT600 in vidéo

- engine screw helicoil (aluminium to steell) for 2 engine screws + 1 inside cover

- ring oil filter sometimes stuck inside, breaking oil cover ears

- clutch axle

- new oil pump + sprocket

- turn oil pump + refil oil tank, to ensure oil flowing

- carbon fiber disk for clutch more grip = more torque

- 6 disques garnies 2 grands (différente taille)

- rotate disks to change use point

- new clutch plate from newer model with a point on arrow alignment

- rear brake mod up to have more ground clearance

https://www.youtube.com/watch?v=yGB1oIcLBnU

- 2nd clutch cable already in place ready to be screwed

- 2nd filter already oiled in sous vide bag